SECTION 015000 - TEMPORARY FACILITIES AND CONTROLS

- GENERAL

      - SUMMARY

        - Section includes requirements for temporary utilities, support facilities, and security and protection facilities.

        - Related Requirements:

          - Section 011000 "Summary" for work restrictions and limitations on utility interruptions.

      - USE CHARGES

        - General: Installation and removal of and use charges for temporary facilities shall be included in the Contract Sum unless otherwise indicated. Allow other entities engaged in the Project to use temporary services and facilities without cost, including, but not limited to, Owner's construction forces, Engineer, occupants of Project, testing agencies, and authorities having jurisdiction.

        - Water and Sewer Service from Existing System: Water from Owner's existing water system is available for use without metering and without payment of use charges. Provide connections and extensions of services as required for construction operations.

        - Electric Power Service from Existing System: Electric power from Owner's existing system is available for use without metering and without payment of use charges. Provide connections and extensions of services as required for construction operations.

      - INFORMATIONAL SUBMITTALS

        - Site Utilization Plan: Show temporary facilities, temporary utility lines and connections, staging areas, construction site entrances, vehicle circulation, and parking areas for construction personnel.

        - Project Identification and Temporary Signs: Show fabrication and installation details, including plans, elevations, details, layouts, typestyles, graphic elements, and message content.

        - Fire-Safety Program: Show compliance with requirements of NFPA 241 and authorities having jurisdiction. Indicate Contractor personnel responsible for management of fire-prevention program.

      - QUALITY ASSURANCE

        - Electric Service: Comply with NECA, NEMA, and UL standards and regulations for temporary electric service. Install service to comply with NFPA 70.

        - Tests and Inspections: Arrange for authorities having jurisdiction to test and inspect each temporary utility before use. Obtain required certifications and permits.

        - Accessible Temporary Egress: Comply with applicable provisions in the United States Access Board's ADA-ABA Accessibility Guidelines and ICC/ANSI A117.1.

- PRODUCTS

      - TEMPORARY FACILITIES

        - Field Offices, General: Prefabricated or mobile units with serviceable finishes, temperature controls, and foundations adequate for normal loading.

- EXECUTION

      - TEMPORARY FACILITIES, GENERAL

        - Conservation: Coordinate construction and use of temporary facilities with consideration given to conservation of energy, water, and materials. Coordinate use of temporary utilities to minimize waste.

          - Salvage materials and equipment involved in performance of, but not actually incorporated into, the Work. See other Sections for disposition of salvaged materials that are designated as Owner's property.

      - INSTALLATION, GENERAL

        - Locate facilities where they will serve Project adequately and result in minimum interference with performance of the Work. Relocate and modify facilities as required by progress of the Work.

        - Provide each facility ready for use when needed to avoid delay. Do not remove until facilities are no longer needed or are replaced by authorized use of completed permanent facilities.

      - TEMPORARY UTILITY INSTALLATION

        - General: Install temporary service or connect to existing service.

          - Arrange with utility company, Owner, and existing users for time when service can be interrupted, if necessary, to make connections for temporary services.

        - Sewers and Drainage: Provide temporary utilities to remove effluent lawfully.

          - Connect temporary sewers to municipal system as directed by authorities having jurisdiction.

        - Water Service: Install water service and distribution piping in sizes and pressures adequate for construction.

        - Sanitary Facilities: Provide temporary toilets, wash facilities, and drinking water for use of construction personnel. Comply with requirements of authorities having jurisdiction for type, number, location, operation, and maintenance of fixtures and facilities.

        - Temporary Heating and Cooling: Provide temporary heating and cooling required by construction activities for curing or drying of completed installations or for protecting installed construction from adverse effects of low temperatures or high humidity. Select equipment that will not have a harmful effect on completed installations or elements being installed.

        - Isolation of Work Areas in Occupied Facilities: Prevent dust, fumes, and odors from entering occupied areas.

        - Electric Power Service: Provide electric power service and distribution system of sufficient size, capacity, and power characteristics required for construction operations.

          - Install electric power service overhead unless otherwise indicated.

        - Lighting: Provide temporary lighting with local switching that provides adequate illumination for construction operations, observations, inspections, and traffic conditions.

          - Install and operate temporary lighting that fulfills security and protection requirements without operating entire system.

        - Telephone Service: Provide temporary telephone service in common-use facilities for use by all construction personnel. Install one land-based telephone line(s) for each field office.

      - SUPPORT FACILITIES INSTALLATION

        - General: Comply with the following:

          - Provide construction for temporary offices, shops, and sheds located within construction area or within 30 feet of building lines that is noncombustible according to ASTM E 136. Comply with NFPA 241.

          - Maintain support facilities until Engineer schedules Substantial Completion inspection. Remove before Substantial Completion. Personnel remaining after Substantial Completion will be permitted to use permanent facilities, under conditions acceptable to Owner.

        - Traffic Controls: Comply with requirements of authorities having jurisdiction.

          - Protect existing site improvements to remain including curbs, pavement, and utilities.

          - Maintain access for fire-fighting equipment and access to fire hydrants.

        - Parking: Provide temporary parking areas for construction personnel.

        - Project Signs: Project signs are not required.

        - Waste Disposal Facilities: Comply with requirements specified in Section 017419 "Construction Waste Management and Disposal."

        - Lifts and Hoists: Provide facilities necessary for hoisting materials and personnel.

          - Truck cranes and similar devices used for hoisting materials are considered "tools and equipment" and not temporary facilities.

      - SECURITY AND PROTECTION FACILITIES INSTALLATION

        - Protection of Existing Facilities: Protect existing vegetation, equipment, structures, utilities, and other improvements at Project site and on adjacent properties, except those indicated to be removed or altered. Repair damage to existing facilities.

          - Where access to adjacent properties is required in order to affect protection of existing facilities, obtain written permission from adjacent property owner to access property for that purpose.

        - Environmental Protection: Provide protection, operate temporary facilities, and conduct construction as required to comply with environmental regulations and that minimize possible air, waterway, and subsoil contamination or pollution or other undesirable effects.

        - Temporary Erosion and Sedimentation Control: Provide measures to prevent soil erosion and discharge of soil-bearing water runoff and airborne dust to undisturbed areas and to adjacent properties and walkways, according to requirements of EPA Construction General Permit or authorities having jurisdiction, whichever is more stringent.

          - Verify that flows of water redirected from construction areas or generated by construction activity do not enter or cross tree- or plant-protection zones.

          - Inspect, repair, and maintain erosion- and sedimentation-control measures during construction until permanent vegetation has been established.

          - Clean, repair, and restore adjoining properties and roads affected by erosion and sedimentation from Project site during the course of Project.

          - Remove erosion and sedimentation controls and restore and stabilize areas disturbed during removal.

        - Stormwater Control: Comply with requirements of authorities having jurisdiction. Provide barriers in and around excavations and subgrade construction to prevent flooding by runoff of stormwater from heavy rains.

        - Security Enclosure and Lockup: Install temporary enclosure around partially completed areas of construction. Provide lockable entrances to prevent unauthorized entrance, vandalism, theft, and similar violations of security. Lock entrances at end of each workday.

        - Barricades, Warning Signs, and Lights: Comply with requirements of authorities having jurisdiction for erecting structurally adequate barricades, including warning signs and lighting.

        - Temporary Egress: Maintain temporary egress from existing occupied facilities as indicated and as required by authorities having jurisdiction.

        - Temporary Enclosures: Provide temporary enclosures for protection of construction, in progress and completed, from exposure, foul weather, other construction operations, and similar activities.

          - Where heating or cooling is needed and permanent enclosure is incomplete, insulate temporary enclosures.

      - OPERATION, TERMINATION, AND REMOVAL

        - Supervision: Enforce strict discipline in use of temporary facilities. To minimize waste and abuse, limit availability of temporary facilities to essential and intended uses.

        - Maintenance: Maintain facilities in good operating condition until removal.

          - Maintain operation of temporary enclosures, heating, cooling, humidity control, ventilation, and similar facilities on a 24-hour basis where required to achieve indicated results and to avoid possibility of damage.

        - Temporary Facility Changeover: Do not change over from using temporary security and protection facilities to permanent facilities until Substantial Completion.

        - Termination and Removal: Remove each temporary facility when need for its service has ended, when it has been replaced by authorized use of a permanent facility, or no later than Substantial Completion. Complete or, if necessary, restore permanent construction that may have been delayed because of interference with temporary facility. Repair damaged Work, clean exposed surfaces, and replace construction that cannot be satisfactorily repaired.

          - Materials and facilities that constitute temporary facilities are property of Contractor. Owner reserves right to take possession of Project identification signs.

          - At Substantial Completion, repair, renovate, and clean permanent facilities used during construction period. Comply with final cleaning requirements specified in Section 017700 "Closeout Procedures."

END OF SECTION 015000