SECTION 017300 - EXECUTION

- GENERAL

      - SUMMARY

        - Section includes general administrative and procedural requirements governing execution of the Work including, but not limited to, the following:

          - Construction layout.

          - Field engineering and surveying.

          - Installation of the Work.

          - Cutting and patching.

          - Progress cleaning.

          - Starting and adjusting.

          - Protection of installed construction.

        - Related Requirements:

          - Section 011000 "Summary" for limits on use of Project site.

          - Section 017700 "Closeout Procedures" for submitting Project Record Documents, recording of Owner-accepted deviations from indicated lines and levels, replacing defective work, and final cleaning.

      - QUALITY ASSURANCE

        - Land Surveyor Qualifications: A professional land surveyor who is legally qualified to practice in jurisdiction where Project is located and who is experienced in providing land-surveying services of the kind indicated.

        - Cutting and Patching: Comply with requirements for and limitations on cutting and patching of construction elements.

          - Structural Elements: When cutting and patching structural elements, notify Engineer of locations and details of cutting and await directions from Engineer before proceeding. Shore, brace, and support structural elements during cutting and patching. Do not cut and patch structural elements in a manner that could change their load-carrying capacity or increase deflection.

          - Operational Elements: Do not cut and patch operating elements and related components in a manner that results in reducing their capacity to perform as intended or that results in increased maintenance or decreased operational life or safety.

          - Other Construction Elements: Do not cut and patch other construction elements or components in a manner that could change their load-carrying capacity, that results in reducing their capacity to perform as intended, or that results in increased maintenance or decreased operational life or safety.

          - Visual Elements: Do not cut and patch construction in a manner that results in visual evidence of cutting and patching. Do not cut and patch exposed construction in a manner that would, in Engineer's opinion, reduce the building's aesthetic qualities. Remove and replace construction that has been cut and patched in a visually unsatisfactory manner.

        - Manufacturer's Installation Instructions: Obtain and maintain on-site manufacturer's written recommendations and instructions for installation of products and equipment.

- PRODUCTS

      - MATERIALS

        - General: Comply with requirements specified in other Sections.

        - In-Place Materials: Use materials for patching identical to in-place materials. For exposed surfaces, use materials that visually match in-place adjacent surfaces to the fullest extent possible.

          - If identical materials are unavailable or cannot be used, use materials that, when installed, will provide a match acceptable to Engineer for the visual and functional performance of in-place materials.

- EXECUTION

      - EXAMINATION

        - Existing Conditions: The existence and location of underground and other utilities and construction indicated as existing are not guaranteed. Before beginning sitework, investigate and verify the existence and location of underground utilities, mechanical and electrical systems, and other construction affecting the Work.

          - Before construction, verify the location and invert elevation at points of connection of sanitary sewer, storm sewer, and water-service piping; underground electrical services; and other utilities.

          - Furnish location data for work related to Project that must be performed by public utilities serving Project site.

        - Examination and Acceptance of Conditions: Before proceeding with each component of the Work, examine substrates, areas, and conditions, with Installer or Applicator present where indicated, for compliance with requirements for installation tolerances and other conditions affecting performance. Record observations.

          - Examine roughing-in for mechanical and electrical systems to verify actual locations of connections before equipment and fixture installation.

          - Examine walls, floors, and roofs for suitable conditions where products and systems are to be installed.

          - Verify compatibility with and suitability of substrates, including compatibility with existing finishes or primers.

        - Proceed with installation only after unsatisfactory conditions have been corrected. Proceeding with the Work indicates acceptance of surfaces and conditions.

      - PREPARATION

        - Existing Utility Information: Furnish information to Owner that is necessary to adjust, move, or relocate existing utility structures, utility poles, lines, services, or other utility appurtenances located in or affected by construction. Coordinate with authorities having jurisdiction.

        - Field Measurements: Take field measurements as required to fit the Work properly. Recheck measurements before installing each product. Where portions of the Work are indicated to fit to other construction, verify dimensions of other construction by field measurements before fabrication. Coordinate fabrication schedule with construction progress to avoid delaying the Work.

        - Space Requirements: Verify space requirements and dimensions of items shown diagrammatically on Drawings.

        - Review of Contract Documents and Field Conditions: Immediately on discovery of the need for clarification of the Contract Documents caused by differing field conditions outside the control of Contractor, submit a request for information to Engineer according to requirements in Section 013100 "Project Management and Coordination."

      - CONSTRUCTION LAYOUT

        - Verification: Before proceeding to lay out the Work, verify layout information shown on Drawings, in relation to the property survey and existing benchmarks. If discrepancies are discovered, notify Engineer and Construction Manager promptly.

        - General: Engage a land surveyor to lay out the Work using accepted surveying practices.

          - Establish benchmarks and control points to set lines and levels of construction and elsewhere as needed to locate each element of Project.

          - Establish limits on use of Project site.

          - Establish dimensions within tolerances indicated. Do not scale Drawings to obtain required dimensions.

          - Inform installers of lines and levels to which they must comply.

          - Check the location, level and plumb, of every major element as the Work progresses.

          - Notify Engineer and Construction Manager when deviations from required lines and levels exceed allowable tolerances.

          - Close site surveys with an error of closure equal to or less than the standard established by authorities having jurisdiction.

        - Record Log: Maintain a log of layout control work. Record deviations from required lines and levels. Include beginning and ending dates and times of surveys, weather conditions, name and duty of each survey party member, and types of instruments and tapes used. Make the log available for reference by Engineer and Construction Manager.

      - FIELD ENGINEERING

        - Identification: Owner will identify existing benchmarks, control points, and property corners.

        - Reference Points: Locate existing permanent benchmarks, control points, and similar reference points before beginning the Work. Preserve and protect permanent benchmarks and control points during construction operations.

        - Benchmarks: Establish and maintain a minimum of two permanent benchmarks on Project site, referenced to data established by survey control points. Comply with authorities having jurisdiction for type and size of benchmark.

          - Record benchmark locations, with horizontal and vertical data, on Project Record Documents.

      - INSTALLATION

        - General: Locate the Work and components of the Work accurately, in correct alignment and elevation, as indicated.

          - Make vertical work plumb and make horizontal work level.

          - Where space is limited, install components to maximize space available for maintenance and ease of removal for replacement.

          - Conceal pipes, ducts, and wiring in finished areas unless otherwise indicated.

        - Comply with manufacturer's written instructions and recommendations for installing products in applications indicated.

        - Install products at the time and under conditions that will ensure the best possible results. Maintain conditions required for product performance until Substantial Completion.

        - Conduct construction operations so no part of the Work is subjected to damaging operations or loading in excess of that expected during normal conditions of occupancy.

        - Sequence the Work and allow adequate clearances to accommodate movement of construction items on site and placement in permanent locations.

        - Tools and Equipment: Where possible, select tools or equipment that minimize production of excessive noise levels.

        - Templates: Obtain and distribute to the parties involved templates for work specified to be factory prepared and field installed. Check Shop Drawings of other portions of the Work to confirm that adequate provisions are made for locating and installing products to comply with indicated requirements.

        - Attachment: Provide blocking and attachment plates and anchors and fasteners of adequate size and number to securely anchor each component in place, accurately located and aligned with other portions of the Work. Where size and type of attachments are not indicated, verify size and type required for load conditions.

          - Mounting Heights: Where mounting heights are not indicated, mount components at heights directed by Engineer.

          - Allow for building movement, including thermal expansion and contraction.

          - Coordinate installation of anchorages. Furnish setting drawings, templates, and directions for installing anchorages, including sleeves, concrete inserts, anchor bolts, and items with integral anchors, that are to be embedded in concrete or masonry. Deliver such items to Project site in time for installation.

        - Joints: Make joints of uniform width. Where joint locations in exposed work are not indicated, arrange joints for the best visual effect. Fit exposed connections together to form hairline joints.

        - Remove and replace damaged, defective, or non-conforming Work.

      - CUTTING AND PATCHING

        - Cutting and Patching, General: Employ skilled workers to perform cutting and patching. Proceed with cutting and patching at the earliest feasible time, and complete without delay.

          - Cut in-place construction to provide for installation of other components or performance of other construction, and subsequently patch as required to restore surfaces to their original condition.

        - Existing Warranties: Remove, replace, patch, and repair materials and surfaces cut or damaged during installation or cutting and patching operations, by methods and with materials so as not to void existing warranties.

        - Temporary Support: Provide temporary support for work to be cut.

        - Protection: Protect in-place construction during cutting and patching to prevent damage. Provide protection from adverse weather conditions for portions of Project that might be exposed during cutting and patching operations.

        - Adjacent Occupied Areas: Where interference with use of adjoining areas or interruption of free passage to adjoining areas is unavoidable, coordinate cutting and patching according to requirements in Section 011000 "Summary."

        - Existing Utility Services and Mechanical/Electrical Systems: Where existing services/systems are required to be removed, relocated, or abandoned, bypass such services/systems before cutting to prevent interruption to occupied areas.

        - Cutting: Cut in-place construction by sawing, drilling, breaking, chipping, grinding, and similar operations, including excavation, using methods least likely to damage elements retained or adjoining construction. If possible, review proposed procedures with original Installer; comply with original Installer's written recommendations.

          - In general, use hand or small power tools designed for sawing and grinding, not hammering and chopping. Cut holes and slots neatly to minimum size required, and with minimum disturbance of adjacent surfaces. Temporarily cover openings when not in use.

          - Finished Surfaces: Cut or drill from the exposed or finished side into concealed surfaces.

          - Concrete: Cut using a cutting machine, such as an abrasive saw or a diamond-core drill.

          - Excavating and Backfilling: Comply with requirements in applicable Sections where required by cutting and patching operations.

          - Mechanical and Electrical Services: Cut off pipe or conduit in walls or partitions to be removed. Cap, valve, or plug and seal remaining portion of pipe or conduit to prevent entrance of moisture or other foreign matter after cutting.

          - Proceed with patching after construction operations requiring cutting are complete.

        - Patching: Patch construction by filling, repairing, refinishing, closing up, and similar operations following performance of other work. Patch with durable seams that are as invisible as practicable. Provide materials and comply with installation requirements specified in other Sections, where applicable.

          - Inspection: Where feasible, test and inspect patched areas after completion to demonstrate physical integrity of installation.

          - Exposed Finishes: Restore exposed finishes of patched areas and extend finish restoration into retained adjoining construction in a manner that will minimize evidence of patching and refinishing.

          - Pavements: Where pavements that are worked on, patch and repair pavement surfaces. Provide an even surface of uniform finish, color, texture, and appearance.

        - Cleaning: Clean areas and spaces where cutting and patching are performed. Remove paint, mortar, oils, putty, and similar materials from adjacent finished surfaces.

      - PROGRESS CLEANING

        - General: Clean Project site and work areas daily, including common areas. Enforce requirements strictly. Dispose of materials lawfully.

          - Comply with requirements in NFPA 241 for removal of combustible waste materials and debris.

          - Do not hold waste materials more than seven days during normal weather or three days if the temperature is expected to rise above 80 deg F.

          - Containerize hazardous and unsanitary waste materials separately from other waste. Mark containers appropriately and dispose of legally, according to regulations.

            - Use containers intended for holding waste materials of type to be stored.

          - Coordinate progress cleaning for joint-use areas where Contractor and other contractors are working concurrently.

        - Site: Maintain Project site free of waste materials and debris.

        - Work Areas: Clean areas where work is in progress to the level of cleanliness necessary for proper execution of the Work.

          - Remove liquid spills promptly.

          - Where dust would impair proper execution of the Work, broom-clean or vacuum the entire work area, as appropriate.

        - Installed Work: Keep installed work clean. Clean installed surfaces according to written instructions of manufacturer or fabricator of product installed, using only cleaning materials specifically recommended. If specific cleaning materials are not recommended, use cleaning materials that are not hazardous to health or property and that will not damage exposed surfaces.

        - Concealed Spaces: Remove debris from concealed spaces before enclosing the space.

        - Exposed Surfaces in Finished Areas: Clean exposed surfaces and protect as necessary to ensure freedom from damage and deterioration at time of Substantial Completion.

        - Waste Disposal: Do not bury or burn waste materials on-site. Do not wash waste materials down sewers or into waterways. Comply with waste disposal requirements in Section 015000 "Temporary Facilities and Controls" and Section 017419 "Construction Waste Management and Disposal."

        - During handling and installation, clean and protect construction in progress and adjoining materials already in place. Apply protective covering where required to ensure protection from damage or deterioration at Substantial Completion.

        - Clean and provide maintenance on completed construction as frequently as necessary through the remainder of the construction period. Adjust and lubricate operable components to ensure operability without damaging effects.

        - Limiting Exposures: Supervise construction operations to ensure that no part of the construction, completed or in progress, is subject to harmful, dangerous, damaging, or otherwise deleterious exposure during the construction period.

      - PROTECTION OF INSTALLED CONSTRUCTION

        - Provide final protection and maintain conditions that ensure installed Work is without damage or deterioration at time of Substantial Completion.

        - Protection of Existing Items: Provide protection and ensure that existing items to remain undisturbed by construction are maintained in condition that existed at commencement of the Work.

        - Comply with manufacturer's written instructions for temperature and relative humidity.

END OF SECTION 017300