Copyright 2014 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 024119 - SELECTIVE DEMOLITION

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

<Double click here to submit questions, comments, or suggested edits to this section.>

      - SUMMARY

        - Section Includes:

          - Demolition and removal of selected portions of building or structure.

          - Demolition and removal of selected site elements.

          - Salvage of existing items to be reused or recycled.

      - MATERIALS OWNERSHIP

        - Unless otherwise indicated, demolition waste becomes property of Contractor.

        - Historic items, relics, antiques, and similar objects including, but not limited to, cornerstones and their contents, commemorative plaques and tablets, and other items of interest or value to Owner that may be uncovered during demolition remain the property of Owner.

          - Carefully salvage in a manner to prevent damage and promptly return to Owner.

      - PREDEMOLITION MEETINGS

        - Predemolition Conference: Conduct conference at a mutually agreed upon location.

      - INFORMATIONAL SUBMITTALS

        - Proposed Protection Measures: Submit report, including Drawings, that indicates the measures proposed for protecting individuals and property and for environmental protection. Indicate proposed locations and construction of barriers if applicable.

        - Schedule of selective demolition activities with starting and ending dates for each activity.

        - Predemolition photographs or video.

      - CLOSEOUT SUBMITTALS

        - Inventory of items that have been removed and salvaged.

      - NOT USED

      - FIELD CONDITIONS

        - Conditions existing at time of inspection for bidding purpose will be maintained by Owner as far as practical.

        - Notify Architect of discrepancies between existing conditions and Drawings before proceeding with selective demolition.

Retain "Hazardous Materials" paragraph below, or remove all references to hazardous materials. Insert scope of article to include asbestos, PCBs, and other materials if required. Coordinate statements with the General and Supplementary Conditions. See "Hazardous Materials" Article in the Evaluations.

        - Hazardous Materials: It is not expected that hazardous materials will be encountered in the Work.

          - Hazardous materials will be removed by Owner before start of the Work.

Retain subparagraph below to cover instances where hazardous materials are unexpectedly found and must be remediated. See the General Conditions for additional requirements on hazardous materials.

          - If suspected hazardous materials are encountered, do not disturb; immediately notify Architect and Owner. Hazardous materials will be removed by Owner under a separate contract.

        - Sale of removed items or materials on-site is not permitted.

        - Utility Service: Maintain existing utilities indicated to remain in service, if applicable, and protect them against damage during selective demolition operations.

          - Maintain fire-protection facilities in service during selective demolition operations.

        - Arrange selective demolition schedule so as not to interfere with Owner's operations.

      - WARRANTY

        - Existing Warranties: Remove, replace, patch, and repair materials and surfaces cut or damaged during selective demolition, by methods and with materials and using approved contractors so as not to void existing warranties.

- PRODUCTS

      - PERFORMANCE REQUIREMENTS

        - Regulatory Requirements: Comply with governing EPA notification regulations before beginning selective demolition. Comply with hauling and disposal regulations of authorities having jurisdiction.

        - Standards: Comply with ASSE A10.6 and NFPA 241.

- EXECUTION

      - EXAMINATION

        - Verify that utilities have been disconnected and capped, as required, before starting selective demolition operations.

Usually retain first option in first paragraph below. OSHA regulations require only that a "competent person" perform an engineering survey before building demolition begins. See Evaluations.

        - Perform an engineering survey of condition of building to determine whether removing any element might result in structural deficiency or unplanned collapse of any portion of structure or adjacent structures during selective building demolition operations.

        - Inventory and record the condition of items to be removed and salvaged.

      - NOT USED

      - NOT USED

      - PROTECTION

Revise "Temporary Protection" Paragraph below to suit Project. Delete if adequately covered in Section 015000 "Temporary Facilities and Controls."

        - Temporary Protection: Provide temporary barricades and other protection required to prevent injury to people and damage to adjacent buildings and facilities to remain.

      - SELECTIVE DEMOLITION

        - General: Demolish and remove existing construction only to the extent required by new construction and as indicated. Use methods required to complete the Work within limitations of governing regulations.

        - Site Access and Temporary Controls: Conduct selective demolition and debris-removal operations to ensure minimum interference with roads, streets, walks, walkways, and other adjacent occupied and used facilities.

        - Removed and Salvaged Items:

          - Clean salvaged items.

          - Pack or crate items after cleaning. Identify contents of containers.

          - Store items in a secure area until delivery to Owner.

          - Transport items to Owner's storage area designated by Owner.

          - Protect items from damage during transport and storage.

In "Existing Items to Remain" Paragraph below, option adds cost. Revise to suit Project.

        - Existing Items to Remain: Protect construction indicated to remain against damage and soiling during selective demolition.

      - CLEANING

Insert other specific disposal, cleanup, or removal requirements to suit Project.

        - Remove demolition waste materials from Project site and dispose of them in an EPA-approved construction and demolition waste landfill acceptable to authorities having jurisdiction.

          - Do not allow demolished materials to accumulate on-site.

          - Remove and transport debris in a manner that will prevent spillage on adjacent surfaces and areas.

Coordinate first subparagraph below with use of elevators, stairs, or building entries permitted by building manager.

          - Remove debris from elevated portions of building by chute, hoist, or other device that will convey debris to grade level in a controlled descent.

          - Comply with requirements specified in Section 017419 "Construction Waste Management and Disposal."

        - Burning: Do not burn demolished materials.

        - Clean adjacent structures and improvements of dust, dirt, and debris caused by selective demolition operations. Return adjacent areas to condition existing before selective demolition operations began.

END OF SECTION 024119